# Dataset description

| 1 | Normal Case |
| --- | --- |
| 1.1 | Normal case – but no order to fulfil, so shuttle runs through |
| 1.2 | Normal case – No socket placed on shuttle |
| Steps for all faults: - Fault is activated, as described below - After 5 s, fault is resolved, but station is not save - After additional 5 s, station is save - Some cases requires moving of the pressure hat to starting position (after additional 5 s) - Some cases have a switch from one mode to auto mode (after additional 5s) |  |
| 2.1 | Area stop activated -> when locking shuttle |
| 2.2 | Area stop activated -> after locking shuttle, pressing hat moves down |
| 2.3 | Area stop activated -> after locking shuttle, pressing hat at bottom |
| 2.4 | Area stop activated -> after locking shuttle, pressing hat moving up again |
| 3.1 | Light curtain broken -> when locking shuttle |
| 3.2 | Light curtain broken -> after locking shuttle, pressing hat moves down |
| 3.3 | Light curtain broken -> after locking shuttle, pressing hat at bottom |
| 3.4 | Light curtain broken -> after locking shuttle, pressing hat moving up again |
| 4.1 | Pressure mat pressed -> when locking shuttle |
| 4.2 | Pressure mat pressed -> after locking shuttle, pressing hat moves down |
| 4.3 | Pressure mat pressed -> after locking shuttle, pressing hat at bottom |
| 4.4 | Pressure mat pressed -> after locking shuttle, pressing hat moving up again |
| 5.1 | Door switch deactivated -> when locking shuttle |
| 5.2 | Door switch deactivated -> after locking shuttle, pressing hat moves down |
| 5.3 | Door switch deactivated -> after locking shuttle, pressing hat at bottom |
| 5.4 | Door switch deactivated -> after locking shuttle, pressing hat moving up again |
| 6.1 | Emergency stop activated -> when locking shuttle |
| 6.2 | Emergency stop activated -> after locking shuttle, pressing hat moves down |
| 6.3 | Emergency stop activated -> after locking shuttle, pressing hat at bottom |
| 6.4 | Emergency stop activated -> after locking shuttle, pressing hat moving up again |
| 7.1 | QBad -> caused by Area stop, without order on shuttle |
| 7.2 | QBad -> caused by Area stop, during order and pressure hat movement |
| 7.3 | QBad -> caused by Light curtain, without order on shuttle |
| 7.4 | QBad -> caused by Light curtain, during order and pressure hat movement |
| 7.5 | QBad -> caused by Door switch, without order on shuttle |
| 7.6 | QBad -> caused by Door switch, during order and pressure hat movement |
| 8.1 | Position sensor of pressing hat defunct -> when locking shuttle |
| 8.2 | Position sensor of pressing hat defunct -> pressing hat moves down |
| 8.3 | Position sensor of pressing hat defunct -> pressing hat at bottom |
| 8.4 | Position sensor of pressing hat defunct -> pressing hat moving up again |
| 8.5 | Position sensor of pressing hat blocked -> by box 1 |
| 8.6 | Position sensor of pressing hat blocked -> by box 2 |
| 9.1-9.4 | RFID Presence defunct -> no RFID tag mounted on shuttle |
| 10.1-10.2 | Shuttle in station sensor not working -> blocked physically by object |
| 11.1 | No connection to Safety I/O -> shuttles are not moving |
| 11.2 | No connection to Safety I/O -> shuttles moving, no order |
| 11.3 | No connection to Safety I/O -> shuttles moving, with order, fault during locking shuttle |
| 11.4 | No connection to Safety I/O -> shuttles moving, with order, fault when pressing hat moves up |
| 12.1 | No connection to control centre -> shuttles are not moving |
| 12.2 | No connection to control centre -> shuttles moving, no order |
| 12.3 | No connection to control centre -> shuttles moving, with order, fault during locking shuttle |
| 12.4 | No connection to control centre -> shuttles moving, with order, fault when pressing hat moves up |
| 13.1 | No connection to PROFINET -> shuttles are not moving |
| 13.2 | No connection to PROFINET -> shuttles moving, no order |
| 13.3 | No connection to PROFINET -> shuttles moving, with order, fault during locking shuttle |
| 13.4 | No connection to PROFINET -> shuttles moving, with order, fault when pressing hat moves up |
| 14.1 | No connection to RFID -> before shuttle moves into station |
| 14.2 | No connection to RFID -> during shuttle moving into station |
| 14.3 | No connection to RFID -> shuttle in station, with order, pressing hat moving down |
| 14.4 | No connection to RFID -> shuttle in station, with order, pressing hat moving up |

# Error description

| 0 | Normal behaviour |
| --- | --- |
| 1 | Station not save |
| 2 | Area stop |
| 3 | Light curtain |
| 4 | Pressure mat |
| 5 | Door safety switch |
| 6 | Emergency stop |
| 7 | Safety components passivated |
| 8 | Position sensor of press broken |
| 9 | RFID Presence not working defunct |
| 10 | Shuttle in Station sensor defunct |
| 11 | no connection to ET200S |
| 12 | no connection to Control station |
| 13 | no connection to PROFINET |
| 14 | no connection to RFID |
| 15 | Station not started in automatic mode |
| 16 | Press not in starting position |
| 17 | software error during main process execution |